| Bill of Materials |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product Name |  |  |  |  |  |  |  |  |
| Team |  |  |  |  |  |  |  |  |
| Part # | Part Name | Qty | Description | Functions | Material | Manufacturing Process | Dimensions | Photo |
| 1 | Leaflet | 18 | Iris leaflet | Directly crimp the stent | Steel | Machining, Cut | Radius 60 mm Angle 20 degrees Fillet radius 5 mm |  |
| 2 | Shoulder Screw | 18 | Metal Securer | Attaches leaflets to front plate | Steel | Purchased | Shoulder- Diameter: 4 mm Threads- M3x0.5 Depth 4.5 mm |  |
| 3 | Nonrotating plate | 1 | Stationary Plate with narrow linear slots | Allows leaflet movements | Metal | Machining, Cut | Outer Diameter 140 mm Inner Diameter 72 mm Slot length 44 mm by 1 mm |  |
| 4 | Rotating Plate | 1 | Rotating Plate with linear slots | Facilitates movement of leaflets from motor | Metal | Machining, Cut | Outer Diameter 140 mm Inner Diameter 72 mm Slot thickness 4 mm |  |
| 5 | Worm Gear | 1 | DB1414A Worm Gear | Actuate Rotating Plate | Bronze | Purchased | Outer Diameter: 158.75 mm Pitch 10 Ratio 30:1 Double-threaded Right-handed |  |
| 6 | Worm | 1 | D1418KRH Worm | Translate Motion of motor to Gear | Unhardened Steel | Purchased | Outer Diameter: 31.75 mm Pitch 10 Double-threaded Right-handed |  |
| 7 | Stepper Motor | 1 | Stepper Motor | Moves the front plate and leaflets | Metal, plastic | Purchased | Frame: 60 mm Motor Length: 83 mm Shaft Diameter: 10 mm |  |
| 8 | Driver | 1 | CVD528BR-K CVD 5-Phase Driver | Controls motor | Metal | Purchased | Power Supply: 24 VDC Current: 2.8 A |  |
| 9 | Load Cell | 1 | 3 Kg Compression Force Sensor | Measures Force output by worm gear | Stainless Steel | Purchased | 3kg Sensitivity: 1.0-2.0 mV/V Voltage: 5-12 V |  |
| 10 | Display screen | 1 | LCD1602 display | Displays Radial Force and Diameter |  | Purchased | LCD1602 |  |